Wheelchair Accessibility Rules DIN 18040

CHECK doors:

* clear width ≥ 90 cm

* clear height ≥ 205 cm

* reveal depth ≤ 26 cm

* handle distance to obstacles ≥ 50 cm

* no threshold or lower door stop, unless technically unavoidable and ≤ 2 cm

CHECK automatic doors:

* frontal swing door: opening direction ≥ 250 cm

* frontal swing door: closing direction ≥ 150 cm

* frontal sliding door: both sides ≥ 150 cm

CHECK corridors:

* slope ≤ 3%

* for corridor length ≤ 10 m, slope may be ≤ 4%

* usable width normally ≥ 150 cm

* reduced width ≥ 120 cm only with defined movement areas and limited conditions

movement area after max. 15 m:

* public buildings: ≥ 180 x 180 cm

* residential buildings: ≥ 150 x 150 cm

* usable width in passages ≥ 90 cm

CHECK wheelchair parking:

* required where wheelchair change/exchange is needed

* required for each dwelling with unrestricted wheelchair use

* parking area ≥ 180 x 150 cm

* additional maneuvering area ≥ 180 x 150 cm

* maneuvering area may overlap with other movement areas

CHECK living rooms and bedrooms:

* barrier-free dwelling: turning/movement area ≥ 120 x 120 cm

* wheelchair-accessible dwelling: turning/movement area ≥ 150 x 150 cm

* movement areas may overlap

* free depth along/in front of furniture:

* barrier-free dwelling: ≥ 90 cm

* wheelchair-accessible dwelling: ≥ 150 cm

* bedroom with bed:

* barrier-free dwelling: ≥ 120 cm on one long side and ≥ 90 cm on the other long side

* wheelchair-accessible dwelling: ≥ 150 cm on one long side and ≥ 120 cm on the other long side

* additional individual space, if required: ≥ 15 m²

CHECK event rooms:

* wheelchair spectator spaces must be provided

* rear/frontal approach: wheelchair space ≥ 90 x 130 cm

* rear/frontal approach: adjacent side circulation width ≥ 150 cm

* side approach: wheelchair space ≥ 150 x 90 cm

* side approach: adjacent side circulation width ≥ 90 cm

* movement and circulation areas may overlap

* companion seats should be placed next to wheelchair spaces

* accessible stage access should be provided, e.g. platform lift or stair lift

CHECK sanitary rooms:

* public buildings: at least one accessible toilet per sanitary facility

* accessible toilet must be integrated into gender-specific areas or provided as a separate gender-neutral toilet

* residential buildings: at least one sanitary room must be barrier-free usable

* swing doors must not open into sanitary rooms

* doors must be unlockable from outside

* wheelchair-accessible sanitary rooms: movement area ≥ 150 x 150 cm in front of all objects and in the shower area

* barrier-free dwellings: movement area ≥ 120 x 120 cm is sufficient

* movement areas may overlap

CHECK wheelchair-accessible WC:

* movement area in front of sanitary objects ≥ 150 x 150 cm

* transfer space next to WC: ≥ 90 cm wide and ≥ 70 cm deep

* transfer space must be accessible from both sides

CHECK elevators:

* Type 2 elevator:

* door clear width ≥ 90 cm

* cabin width ≥ 110 cm

* cabin depth ≥ 140 cm

* movement area in front of elevator shaft door ≥ 150 x 150 cm

* movement area may overlap with circulation routes only if passing a waiting wheelchair user is still possible

* if needed, circulation route should be widened by ≥ 90 cm

* distance to opposite downward staircase ≥ 300 cm

CHECK stairs / vertical accessibility:

* stairs may not be the only vertical connection in publicly accessible buildings

* stairs must be supplemented by elevators or ramps

CHECK ramps:

* entrance circulation areas with slope ≥ 3% must be designed as ramps

* for entrance circulation area length ≤ 10 m: longitudinal slope ≤ 4%

* public ramps: longitudinal slope ≤ 6%

* public ramps: cross slope = 0%

* clear ramp width ≥ 120 cm

* ramp flight length ≤ 600 cm

* horizontal movement area at start and end ≥ 150 x 150 cm

* intermediate landing required after 600 cm ramp length

* intermediate landing length ≥ 150 cm

* no downward staircase directly in the continuation of a ramp

* sufficient movement area is required in front of doors opening toward the ramp

* with two ramp flights, maximum height difference ≈ 72 cm

* for height difference > 100 cm, elevator or lifting platform is recommended